WIH & WRH Louvered Penthouse Specification

## Construction Features

Louvered penthouses shall be model WIH or WRH as manufactured by Greenheck (www.greenheck.com). Louver shall be 4 inch deep with 0.081 inch extruded aluminum wall thickness and mitered corners. Unit shall have a removable aluminum cover for internal access and shall be lined with fiberglass insulation to prevent condensation. Cover shall be securely fastened to the unit by corrosion-resistant fasteners. Finish of all materials shall be mill (none) unless specified from available options.

## Performance Requirements

Louvers shall be mounted 4 inches (102 mm) from the throat for maximum weather protection. Louver design shall result in low pressure drops, low hood heights and maintain maximum weather resistance.

Manufacturer shall provide third-party verified Environmental Product Declaration specific to the submitted louver model.

## Available Options and Accessories (Specify as Required)

- Tie down points

- Roof curb

- Hood insulation

- A variety of damper models

- A variety of architectural finishes

  - 70% Kynar Paint/100% Fluoropolymer (AAMA 2605)

  - 50% Kynar Paint (AAMA 2604)

  - Baked Enamel Paint (AAMA 2603)

  - Prime Paint

- A variety of bird screen options

  - 0.75 inch x 0.051 inch flattened expanded aluminum unless otherwise selected

  - 0.5 inch x 0.063 inch square mesh aluminum bird screen

- A variety of insect screen options

  - 18 x 16 mesh x 0.011 inch diameter mesh aluminum wire insect screen

  - 18 x 18 mesh x 0.009 inch diameter stainless steel wire insect screen